Adapted from BC CDC PHSA Lab Manual

Background

Nasopharyngeal swabs are used for the detection of respiratory viruses such as RSV, influenza virus A & B or parainfluenza virus.  A properly collected viral swab (one nare is sufficient) is necessary for detection of these organisms.

Assemble supplies:

- Universal Transport Medium (UTM) Kit (red top viral transfer container). Check expiration date. Personal protection equipment (i.e., mask, gloves, eye protection, gowns). Requisition and label, biohazard bag. Note: if no red top UTM kits are available, the blue top viral transfer may be used.

- Explain procedure to patient.

- Wash hands. Put on appropriate personal protective equipment (at a minimum, gloves and facemask) to   protect   yourself   in case the patient coughs or sneezes while you are collecting the specimen.

- If the patient has a lot of mucous in his/her nose, this can interfere with the collection of cells. Ask the patient to use a tissue to gently clean out visible nasal mucous before a swab is taken. Influenza viruses are located in cells that line the surface of the inner nose. The virus is not found in the mucous discharge.

- Seat patient in a comfortable bed. It is best if the patient is placed in a high-fowler’s position in bed with the back of the head supported. It may be necessary to have a second person available to assist with collection.

- Swab collection

Nasopharyngeal swab

- Enter a flexible swab several centimeters with a slow, steady motion along the floor of the nose (straight back, not up the nose) until the posterior nasopharynx has been reached (distance from nostrils to external opening of ear).

- Place finger on the tip of the patient/resident’s nose and depress slightly.

- Once resistance is met (the swab should pass into the pharynx relatively easily), rotate the swab several times and withdraw the swab.

- Break off top of swab (it will snap off).

- Place in transport medium.

- Remove personal protective equipment, wash hands.

- Ensure the specimen is labeled and transport to the laboratory with completed requisition.